# SAN Server Configuration using iSCSI Protocol

On Server Side

- For this Configuration Lab, you will be requiring total 2 HDDs, one will behave as a driver for iSCSI controller & other one would be used as block device on client side. I will be using /dev/sda & /dev/sdb for this lab.

- Assign a MSDOS style Partition table to the HDD (/dev/sdb) & start creating partitions into it. This Block Device will be shared in the network.

- For Configuration of SAN Server, you require a package called targetcli, check if it installed on your system or not. If not present, install it.

- Enable the Service called target.service, in order to start the configuration for SAN server

- Enter inside the Tagetcli utility in order to configure SAN server.

- List all the different directories inside targetcli utility

- Here we have converted our partition (/dev/sdb1) into virtual block storage device.

- Here we have created a driver for our iSCSI Controller & its Image is saved at /root/driver.img which has size of 100MB (this size is more than enough for the driver img)

- Verify Block Device & Driver that is been created.

- In this step we are creating a Target inside the Target Server (SAN Server) & assign an IQN (iSCSI Qualified Name) to this target.

- Verify if the target is Created Perfectly using the ls command.

- Now delete the Default Network Portal since it is not our IP address here we are mapping the created IQN with Server’s IP address.

- Create a new network portal for our own IP address

- Now Create Logical Unit Numbers, inside each LUNs reside the Block device and the image driver.

- Configure Access Control Lists, & give access to iSCSI initiator IQN i.e. Client Machine. Remember when there is a SAN server in the network, the client & the server communicate using IQN (iSCSI Qualified Name)

- Verify if the ACLs are configured perfectly.

- Set the Authententication Parameters (userid & password) for iSCSI initiator IQN i.e. Client Machine.

- Verify if iscsi directory is configured perfectly.

- Verify if everything is configured perfectly onto SAN server i.e. iSCSI target

- Exit from the targetcli interface

- After the configuration is done restart the target service.

- Now disable the firewall service

On Client Side

- Make sure iscsi-initiator-utils package is present on the client machine if not then install it.

- Enter the IQN for this iscsi initiator i.e. Client Machine, inside this file location below

- Using the below command you are letting the initiator know about the target (SAN Server)

- After executing the above command, a file will be created inside /var/lib/iscsi. Inside the file add the below lines (It means using this Authentication info. contact the iscsi target)

- Restart the below 2 services after the following configurations has been done.

- Verify using lsblk command that you can see the driver (/dev/sda) and the block device (/dev/sdb) on the Client Machine. This is the flexibility that we get from SAN server, we can configure the block device & start creating partitions based on our choice.

- Assign the partition table to the block device & start creating partitions into it.

- Format the partitions created in order to be in use for storage purpose.

- Create 2 empty directories in order to mount these disk partitions.

- Now mount the disk partitions on respective mount points.

- Create some data into the directory.

You can see that using SAN Server, the block devices are shared in the network & on Client side these block devices can be used based on user’s choice.